Marcel’s Project – Design Criteria for Bioreactor Chamber

1. Redesigning the Casing

- Improve the architecture of the bioreactor casing through computer-aided design and prototyping.

- The design should allow for integration into external mechanical loading systems.

- The design should allow for force transmission from mechanical loading systems.

- The design should enable easy chamber assembly.

- The design should enable independent inlet/outlet pathways for each compartment where flow can occur both inside fibers (intracapillary flow) and in the surrounding space (extracapillary flow) (Eghbali et al., 2016).

- Ensure structural integrity during mechanical loading.

- Ensure leak-free operation and uniform flow during perfusion.

- Ensure sufficient space and geometry for fiber potting and sealing.

2. Fiber Potting

- Develop a reproducible assembly workflow for fiber potting.

- Use computer-aided design and prototyping to improve the potting design.

- The workflow should allow for easy incorporation of fibers into the bioreactor casing.

- The workflow should allow for clean, consistent results.

- Ensure limited fiber curving and even resin distribution.

- Avoid resin infiltration into fibers and resin migration along fibers.

- Ensure design compatibility with post-potting fiber stretching.

- Ensure structural integrity during fiber perfusion.

Papers

Nie, M. et al. (2025) ‘Scalable tissue biofabrication via perfusable hollow fiber arrays for cultured meat applications’, Trends in Biotechnology, 43(8), pp. 1938–1960. doi:10.1016/j.tibtech.2025.02.022.

Hou, Y. et al. (2022) ‘The application of hollow fiber cartridge in biomedicine’, Pharmaceutics, 14(7), p. 1485. doi:10.3390/pharmaceutics14071485.

Diban, N. and Stamatialis, D. (2014) ‘Polymeric hollow fiber membranes for bioartificial organs and tissue engineering applications’, Journal of Chemical Technology &amp; Biotechnology, 89(5), pp. 633–643. doi:10.1002/jctb.4300.

Eghbali, H. et al. (2016) ‘Hollow fiber bioreactor technology for tissue engineering applications’, The International Journal of Artificial Organs, 39(1), pp. 1–15. doi:10.5301/ijao.5000466.